TECHNICKÁ   UNIVERZITA  V LIBERCI

Hálkova 6, 461 17 Liberec 1, CZ

Fakulta mechatroniky a mezioborových inženýrských studií

Control Systems in Mechatronics

INTRODUCTION

TO PARAMETER ESTIMATION

(Working version)

Student text

Doc. Ing. Osvald Modrlák, CSc.

March, 2009

Institute of Systems Control and Reliability Management (RSS)

1	Introduction	2

2	Fundamental concepts	3

2.1	Mathematic model for experimental identification	3

2.2	Model of the controlled process	4

3	Parameter estimation of a transfer function with given structure	6

3.1	Identified process, input and output signals	6

3.1.1	Identified process	6

3.1.2	Output signal	7

3.2	Identification measuring and data acquisition	8

3.2.1	Input test signals	8

3.3	Structure of parameter estimation	10

3.4	Parametric estimation of the transfer function by given structure	11

3.4.1	Structure of an ON-line parameter estimation with LTI  model	11

3.4.2	Structure of an Off-Line parameter estimation with LTI model	12

3.4.3	Model structure	12

3.4.4	Criterion	14

3.4.5	Optimization strategy	15

3.5	Searching for the optimal model	17

3.5.1	Searching for the optimal structure of the transfer function	17

3.5.2	Experimental identification procedure	18

3.6	Movement to the working point	19

3.7	Model Reduction	20

3.8	Standardization of input and output signals	21

4	Tools for parameter estimation	22

4.1	System Identification Toolbox “Ident”	22

4.2	MATLAB Program for parameter estimation	22

4.2.1	Flow-process diagram	22

4.2.2	Program and function in MATLAB	23

5	Introduction to using MATLAB	24

5.1	Desktop Tools and Development Environment	26

5.2	Workspace, Editor, Search path and File Operations	27

5.2.1	Workspace operations	27

5.2.2	Editor and Command Window operation	27

5.2.3	Search part and File operation	27

5.2.4	Select function size, length, roots, conv, disp, pause	28

5.3	LTI models	29

5.3.1	Creating of Some Linear Models	29

5.3.2	Data Extraction	31

5.4	Time Domain Analysis	32

5.5	Some basic plotting function	36

5.6	Opening, loading, saving files	42

5.7	Programming-flow control	47

Literatura	51

This student text offers a comprehensive overview of parameter estimation for linear dynamic systems with accent on an appropriate balance between theoretical essentials and engineering practice in identification. The student’s text presents special part of empirical identification that is referred to parameter estimation.

- Introduction

Week 1: Parametric and non parametric identification, model of the controlled process, identified process, input and output signals, design of identification measure, measuring noise signals.

Analyses, syntheses and modeling of dynamic systems and every optimization need mathematical models. For our use we will accept the definition of mathematical model by Eykhoff

A mathematical model is a representation of the essentials aspects of an existing

(or system to be constructed) which presents knowledge of that system in usable form.

Each mathematical model represents always inevitable simplification and uncertainty of the modeled real process. On principle it is yet caused by choosing the mathematical model. The models can be classified in:

- Model with lumped or distributed parameters. When the real dynamic system is described by ordinary differential equations, this is referred to as model with lumped parameters. Model that describes the modeled system with partial differential equations is the model with distributed parameters.

- Linear or nonlinear models are describing the dynamic system with linear or nonlinear equation.

- Deterministic or stochastic models. Deterministic model is one in which every set of variables states is uniquely determined by parameters in the model and by sets of these variables. Therefore the deterministic model has the same outputs for a given set of initial conditions. In a stochastic model randomness is present, and variable states are not described by unique values, but rather by probability distributions.

- Linear time invariant model are describing the dynamic system with linear differential equations with constant parameters.

Let us suppose a real system to be modeled with an input and output shown in the Fig.1a. Mathematical modeling of a real system generally depends on a priori information about the modeled system. What shell we consider as a priori information?

Input

Output

Black-box model

Input

System

no a priory information

available

Output

Fig.1a Modeled system         Fig. 1b) Mathematical model

Generally a priori information contains knowledge, facts or principles about the process that are known.

A posteriori information contains knowledge after analysing, thinking  sth by starting from known facts with “a priori information”and making experiments.

They may come in forms of knowing the type of functions relating different variables, the model structure, types of non-linearity, order of the systems. The operating experience provides a priori information about operating plant condition, limitation on the input perturbation, level of measured noise and so on. Usually it is preferable to use as much a priori information as possible to make the model more accurate. From this point of view the experimental modeling is classified into black-box or white- box model. A black-box model is related to a system of which there is no a priori information available.

A white-box model is related to a system where all necessary information is available. Practically all systems are somewhere between the black-box and white-box models.

- Fundamental concepts

  - Mathematic model for experimental identification

Engineers can obtain the mathematical model by applying fundamental physical principles (theoretical modeling) or by an empirical identification (experimental modeling).

By applying fundamental physical principles the structure as well as parameters of the models is obtained.

The empirical identification is based on data processing of measured signals. This approach usually necessitates a priori information about the structure of the model. The identification is using parametric and non-parametric models.

Parametric models have the model structure a priori specified. The model structure is considered to be the order and the type of differential or difference equation, (linear, non-linear differential or difference equation, type of the non-linearity and so on), or system equations, transfer function with chosen powers of numerator and denominator, or by chosen transfer matrix with m row and n columns.

Parametric models represent from the mathematic point of view equation, equations systems and algebraic equations, which comprise explicitly the coefficients of the equations and formulas. These coefficients are generally referred to as parameters of mathematical models. The parameters are to be determined.

Parametric models

Example 1

a) y''+α⋅(y')2+β⋅y=u,  nonlinear differential equation, parameters α,β

b) T⋅y+y=K⋅u, 		          linear differential equation, parameters T,K

c) x1=x2x2=g−1M(x3)x12x3=−RLx3+1Lu

Nonlinear state equation, parameters

g,M,R,L

Input	       u

Non-parametric models differ from models in that the model structure is not specified a priori but is instead determined from data.

The non-parametric models represent a functional relation between selected input and output. This relation is interpreted geometrically as plotted curves or it is printed as summary sheet (table).

Non-parametric models describe usually transient and impulse response or frequency responses as a plot or in the table form. The parameters are contained implicitly in function relations.

  - Model of the controlled process

A structure of controlled process with its elements is shown in Fig.2.2

yf(t)

AC

A

TP

S

A   … Amplifier

AC…  Actuator

TP…  Technological Process

S  …   Sensors

Controlled Plant /( Process)

uf(t)

u ... Input/Actuating or

Manipulated variable

yf .. Controlled variable

/output

y ... Measured control variable

d … Disturbance

v … Noise signal on

Measuring device

Measuring devices

Fig. 2.2 Scheme of a controlled plant/process with measuring devices

Actuating/Manipulated variable u(t) – is a single input variable that can be adjusted by controller or available for manual experiments.

Controlled variable yS(t)  – is the process selected output variable to be controlled.

Measured control variable y(t) – is the measured control variable.

Disturbances d(t): In addition to actuating variable the controlled plant has inputs, which change the measured control variable. These inputs are called disturbances.

Disturbance - Noise signal on measuring devices n(t) – is a stochastic signal.

Extended controlled plant/process consists of plant/process and measuring devices (see Fig 2.3.) The plant contains transducers, transmitters, amplifiers and actuator. The measuring device embodies sensing elements, transducers, transmitters or sensors.

n

Plant

(Process)

Measuring      devices

Fig. 2.3 Extended controlled plant

Extended Plant/process

Fig 2.4a,c Model structures of

extended plant/process

Fig 2.4b Model structure of controlled process approximated by P(s), H(s) transfer functions.

According to the Norm DIN 19221 two linear time invariant models can be defined for controlled plant proposing that the disturbancesn(t),d(t)=0:

- An extended controlled plant model determined by the transfer function (Fig. 2.4a). The transfer function notation is simplified by lumping all instrumentation and process dynamic into one term FU(s). It can be written

FU(s)=YU(s)U(s)=B(s)A(s)             (2 – 1)

Where, YU(s)=LyU(t),U(s)=Lu(t),A(s),B(s) are polynomials,

yU(t)… is the relating output to input u(t) .

A linear model that approximates the controlled plant by two transfer functions P(s),H(s)is shown in the Fig.2.4b. The transfer function P(s)  approximates the dynamic behaviors of the plant (without the measuring devices) and H(s) approximates the dynamic behaviors of the measuring devices.

b)

Extended Plant/process

Fig 2.4a,c Model structures of

extended plant/process

Fig 2.4b Model structure of controlled process approximated by P(s), H(s) transfer functions.

U(s)

yU(t)

The relationship between the disturbance d(t)and the measured control variable supposing that n(t),u(t)=0 (shown in Fig.2.4c) can be described by the transfer function (C(s) is polynomial)

FD(s)=YD(s)D(s)=C(s)A(s)			(2 – 2)

Fig. 2.5 Model structure of an extended

plant with modeled disturbance

Under these conditions the model structure of an extended controlled plant with modeled disturbance d is shown in the Fig.2.5. The output

of the model is

y(t)=yU(t)+yD(t).

The output of the system for joint action of

manipulated variable and of the disturbance

is equal to

Y(s)=FU(2  3)    (s)⋅U(s)+FD(s)⋅D(s)= =B(s)A(s)U(s)+C(s)A(s)D(s) (2−3).

- Parameter estimation of a transfer function with given structure

It is known that the modern methods of analysis, syntheses and modeling of linear dynamic systems use very often as mathematical model transfer functions.  The accent is laid on the approach that yields the parameters transfer function of linear continuous plant. This approach is referred to as model reference or model matching method.

  - Identified process, input and output signals

### Identified process

The identified process is a controlled plant shown in the Fig.3.1.The process has three inputs: manipulated variable u(t), disturbance d(t) and noise on measuring devices n(t). The output is the measured control variable.

n

Plant

(Process)

Measuring      devices

Fig. 3.1 Identified process

n(t)

Identified

process

u(t)

yU(t)

Fig.3.2a The effect of manipulated

variable

y(t)

y(t)

Identified

process

d(t)

yD(t)

Fig.3.2b The effect of disturbance

variable

y(t)

It is supposed that the effect of manipulated variable u(t) and  disturbance d(t) can be shown by the scheme in the Fig.3.2a,b.

The measured output signal y(t) contains the dynamic effects of manipulated variable u(t), disturbance d(t) and the added noise signal on measuring devices n(t) that is shown in the Fig.3.3a.

In the Fig.3.3b is shown the approximation of the identified process by two transfer functions

Figure 2.  Model of a feedback control system

FU(s) and FD(s). The parameter and the structure of the transfer functions FU(s), FD(s)  must be found by the parameter estimation from the measured input and output signals.

u(t)

y(t)

Identified

process

n(t)

yD(t)

Fu(s)

Fd(s)

u(t)

d(t)

n(t)

y(t)

yD(t)

yu(t)

Fig. 3.3b Model of the identified  process

Fig. 3.3a Identified  process

    - Output signal

Output signal in the Fig.3.3b can be expressed

y(t)=yU(t)+yD(t)+n(t),	(3.1 – 1)

The measured noise n(t) consists of three components: v,χ,λ

n(t)=v(t)+χ(t)+λ(t)	(3.1 – 2)

0

(t)

High frequency – quasi stationary stochastic signal- component ν(t)

0

t

(t)

Low frequency – non stationary stochastic signal-

component χ(t)

(t)

t

Component of the noise signal with unknown source

λ(t)

0

t

Fig.3.5 A realization of the components ν(t), χ(t), λ(t) of the measured noise n(t)

For parameter estimation is supposed that the output signal contain only the high frequency signal v(t). This means the non stationary signal χ(t) must be zero and the additive signal λ(t) must be zero or must be removed.

  - Identification measuring and data acquisition

    - Input test signals

For an identification measure the static characteristic has to be measured that give the information about static behavior of the system and about the linearity of the system.

As an input test signal we are using N step signals-measuring cycles that have constant time of measuring cycles TC. During each measured cycles their amplitudes are constant. It is recommended that the time period TC is so long that the steady state of the measured variable is reached. Each measuring cycle has constant number M of measured samples u(t),y(t) (see Fig.3.4a). The first step u1 must excite the system to the working point.

NON LINEAR STATIC

CHARAKTERISTIC

uh=1,3

ud=0,7

u0=1

Working point

(u0,y0 )=(1,10)

LINEAR STATIC

CHARAKTERISTIC

y(∞)

0,5

1,0

1,5

-0,5

-1,0

u(∞)

u0=0

y0

Starting point and identification time

u0

u1

u2

u3

INPUT STEP SIGNAL

u(t)

Time  of

measuring

cycle TC

y1k

y0

y2k

y3k

Fig.3.4a Choosing of working point at the static characteristic, input step signal and its response

Linear time-invariant (LTI) systems have the static characteristic as a straight line which goes through the system of coordinates. Therefore for a LTI model this requirement must be fulfilled and so, that data for identification data processing of measured input and output must be transformed to the working point. The transform equations are

ΔuID(t)=u(t)−u0,  ΔyID(t)=y(t)−y0,	(3.2 – 1,2)

where

ΔuID(t), ΔyID(t)= Ordinates that are input to identification processing,

u(t), y(t)		    = Measured input and outputs,

u0, y0  		    = Coordinates of input working point.

After the transform the transformed data have the working point y0 = u0 = 0. Practically on the basis of the measured static characteristic the working point must be selected. The chosen working point shall lie in the linear region of the static characteristic. The variation of input is to be carried out around the linear part of the working point as shown in the Fig. 3.4b.

Fig. 3.4b Choice of working points on a measured static characteristic

u0

u0

y0

y0

Linear part around the working point

Linear part around the working point

The transfer function can be determined as

FU(s)=∇YID(s)∇UID(s)=B(s)A(s),			(3.2 – 3)

Where the B(s), A(s) are the polynomials, which have degrees m,nand m<n respectively.

  - Structure of parameter estimation

Week 2: Model structure, parameter estimation of the transfer functions with least squares method and chosen structure based on the input/output measuring of the real plant, chosen criterion and appropriate optimization strategy. ON - line and OFF - line parameter estimation.  Types of model structures, criterion, optimization and flow process diagram of identification MATLAB program. Model order reduction, model verification.

The parameter estimation can be performed direct on the controlled plant so, that the measuring and data processing are carried out immediately. Such parameter estimation is called ON-line.

In the case if the identification measure is separated from the data processing then is referred to as OFF-line.

Structure of an ON-line parameter estimation

The basic idea of parameter estimation is based on the parallel model that has adjustable parameters, what is depicted in the Fig.3.6.

It is supposed that by the identification measure the input signal u(t) is a determined input test signal and all other inputs as disturbance d(t) is constant or zero.

u(t)

Fig.3.6 Structure of parameter estimation

Change of model parameters X

Adjustable

parameter

model

Criterion

J(x)

Identified

Plant/Process

Optimization

strategy of

Parameter

Estimation

The identified plant and the parallel model have the same input - the manipulated variable u(t). The model has adjustable parameters that are comprised in the parameter vector X. By starting the parameter estimation the vector X must contain the initial parameters that guarantee the stability of the parameter model X=X0.

Of course the plant output y(t) depends only on the input in the working point. But the model output depends not only on the input signal but as well on the vector parameters X.

Outputs of the plant and model are compared and the outputs error Δy(t,X) is the input to the criterion block.

The criterion value J(x) is evaluated in the criterion block, which creates the input to the optimization strategy block. The optimization block calculates the new parameter vector X that is transferred to the model.

Now the parameter estimation process is repeating until the criterions for the stop are not achieved.

  - Parametric estimation of the transfer function by given structure

The identification methods for parametric models are based on the assumption that a priory information about the model structure is known or can be chosen. If the assumed model structure matches the process structure, higher model accuracy can be achieved by using statistical regression method.

;

    - Structure of an ON-line parameter estimation with LTI  model

An identification measure for an LTI model must be performed around the chosen working point on the static characteristic. The working point is defined by the pair [u0,y0]. The estimation process is working with

∇u(t)=u(t)−u0			(3.4-1)

∇y(t)=y(t)−y0			(3.4-2)

The structure an ON-line parameter estimation is depicted in the Fig.3.7

y0

u(t)

y(t)

Identified

Plant/Process

∇y(t)

Δ=∇y−∇yM

Adjustable

LTI

model

u0

∇u(t)

∇yM(t,X)

Change of model parameters X

J(x)

Optimization

strategy of

Parameter

Estimation

Criterion

J(x)

Fig.3.7 Structure of an ON-line parameter estimation with LTI model

The adjustable LTI model has the input∇u=u(t)−u0 and the output ∇yM(t,X). The output error is determined

Δ=∇y−∇yM=∇y(t)−∇yM(t,X)=Δ(t,X).       (3.4-3)

    - Structure of an Off-Line parameter estimation with LTI model

By ON-line parameter estimation the input and output from the measure are saved in files. The structure is shown in the Fig.3.8.

Adjustable

LTI

model

Criterion

J(x)

Change of model parameters X

Optimization

strategy of

Parameter

Estimation

Data from file u(t)

Data from file y(t)

Fig.3.8 Structure of an OFF-line parameter estimation with LTI model

The adjustable LTI model has the input∇u=u(t)−u0 and the output ∇yM(t,X). The output error is determined Δ=∇y−∇yM=∇y(t)−∇yM(t,X)=Δ(t,X).

    - Model structure

Suppose that the dynamic of the controlled plant is approximated by a linear time invariant mathematical model in the form of a transfer function. The transfer function is defined as the Laplace transform of the output variable y(t), divided by the Laplace transform of the input variable u(t), with all initial condition equal zero.

FU(s)=YU(s)U(s)=∇Y(s)∇U(s)=bmsm+⋯b1s+b0sn+an−1sn−1+⋯a1s+a0=B(s)A(s),

(3.4-4)

where B(s) and A(s) describe the numerator and denominator polynomials,

n>m, the system is called an n - th order system.

Transfer function with dead time has the form

FU(s)=YU(s)U(s)=∇Y(s)∇U(s)=bmsm+⋯b1s+b0sn+an−1sn−1+⋯a1s+a0e−s⋅Td. (3.4-5)

Generally the structure of the transfer function is defined by the powers n,m of the polynomials A(s), B(s) and by the dead time Td .

The coefficients of the transfer function depend only on the structure - powers n,m and the dead time Td and on the dynamic behavior of the system.

It is known that the transfer function with real roots of the polynomials A(s), B(s) can be written using its time constants in the form

FU(s)=YU(s)U(s)=∇Y∇U=bmsm+⋯b1s+b0sn+an−1sn−1+⋯a1s+a0⋅e−s⋅Td= =K(1+TB1s)⋯(1+TBms)(1+T1s)(1+T2s)⋯(1+Tns)⋅e−s⋅Td(3.4-6)

Some examples of available transfer function structures are described below.

1) F0s=Ke−TDsτs+1,

… 1th order system with time delay

2) F1s=Kτ1s+1τ2s+1,

… 2th order system overdamped

3) F2s=Kτs+1n,

…  n-th order system critically damped

4) F3s=Kτ1s+1τs2+2ξτs+1

…underdamped oscillating

5) F4s=Kτ1s+1τ2s+1τ3s+1

…3th order overdamped

6) F5s=b0s4+a3s3+a2s2+a1s+a0,

X = [b0, a0, a1, a2, a3] parameters vector

7) F6s=b0+b1ss4+a3s3+a2s2+a1s+a0,

X = [b0, b1, a0, a1, a2, a3],

parameters vector

For example we chose 2nd order transfer function in the form with time constants and gain.

F1(s)=K(T1s+1)(T2s+1)=x(1)[x(2)s+1][x(3)s+1]=B(s)A(s)(3.4-7)

where x(1),x(2),x(3) are the unknown parameters.

    - Criterion

As criterion is used the integral of square of the outputs error Δy(t,X)  in the form

J2=J2x=0TMΔ(t,X)2dt=0TMyt−yMt,X2dt,		(3.4-8)

where y(t), yM(t,X) denote the plant and model outputs,

X    	      = parameter vector,

TM            =Time of measure.

The integral of square of the outputs error can be approximated by the sum of outputs error squared Δy(i,X)

J2(X)=i=1NΔ(i,X)2=i=1N[y(i)−yM(i,X)]2		(3.4-9)

Where y(i), yM(i,X) denote the discrete plant and model outputs,

N=  number of the measured samples.

The task is to find the minimum of the unconstrained multivariable functions

minXJ2x=0TMΔ(t,X)2dt=0TMyt−yMt,X2dt

minXJ2(X)=i=1NΔ(i,X)2=i=1N[y(i)−yM(i,X)]2

    - Optimization strategy

For the optimization task to find minimum of unconstrained multivariable function is using derivative-free Nelder-Mead Simplex method.

The method uses the concept of a simplex, which is a polytope of n+1 vertices in n dimensional space. On the vertices are calculated the value of the criterion and the vertices are called as test point.  For example it is a triangle on a plane, a tetrahedron in three-dimensional space and so forth. The polytop is defined by the matrix D [n x (n+1)]

1        2         3       . . .      n+1

Fig.3.9 Polytop (2x3)

1x

2x

3x

D=x,…,x=0d1d2⋯d20d2d1⋯d2⋮0d2d2⋯d20d2d2⋯d1

d1=tn2n+1+n−1            d2=tn2n+1−1

For example: the matrix D for two variables has dimension 2 x 3 and is equal

D=00,9650,25900,2590,965 .

1             2                 3

By start the method calculates in all vertex points (1, 2, 3) the value of the criterion J1(1x), J2(2x), J3(3x). Melder-Mead method then generates a new test position by extrapolating the behaviour of the objective function measured at each test point arranged as a simplex.

The algorithm then chooses to replace one of these test points with the new test point and so the algorithm progresses. For this optimization we are using the function “fminsearch” from optimization Toolbox. The function is complete described in the help but for our quick use we give some basic remarks

Find local minimum of unconstrained

Function fminsearch

multivariable function using derivative-free method

x = fminsearch ('fun',x0)

x = fminsearch ('fun',x0,options)

Syntax

fmins('fun',x0)

'fun'		the function to be minimized. It is a function that accepts a vector x and return

a scalar f . It can be an M-file function or an anonymous function

x0		a starting vector of initial parameter estimate

x		is the result vector

options	is a structure that provides control  of optimization with specific conditions

defined by “options” structure. Use the “optimset” to set the options the values

of these fields in the options structure “OPTIONS”.

String

‘MaxFunEvals’	Maximum number of function evaluations allowed

‘MaxIter’		Maximum number of iterartions allowed

‘TolFun’		Termination tolerance on the function value

‘TolX’		Termination tolerance on x

Example of “optimset’:

OPTIONS=optimset(‘MaxIter’,1000, ‘TolX’, 1e-2)

## Searching for the optimal model

    - Searching for the optimal structure of the transfer function

Searching for the optimal structure of the transfer function is supposed as finding such a structure of the model, which gives us the best agreement between output from model yM(t) and the measured output from the plant.

Verification of the model is supposed as the act of proving the correctness of model output yM(t) to the measured plant outputy(t) with respect to the identification input signal u(t).

For needs in this lecture we are limiting the verification to comparison of the model yM(t) .

By the practical parameter estimation the structure of transfer function is changed and the 7

judgment of agreement is passed between model and identified plant. The judgment is subjective and the following change of the model structure depends on experience of the engineer.  We can give follow advices for model structure optimization:

- Check the steady states of model and identified plant.

- Check the transient parts of the responses that depend on order of the systems. By non minimum systems depend as well as on the degree of the nominator of the transfer function. The nominator degree shell bee small as possible.

- You must ensure that by a oscillating system the denominator of the transfer function contains at least one term [(τs)2+2ξτ⋅s+1].

- Chose degree of nominator zero and by changing step by step the degree of the denominator (1, 2, 3, 4) you shall prove how matches the responses of the model with the identified plant. It is possible to improve the model by choosing the degree of nominator 1 or maximal 2.

- Chose the time delay, when the identification measure of the plant matches with a time delay or you will make the order of the model smaller.

- Check the performance criterion. Attention! The value of the criterion cannot be taken absolutely and separately. We are always searching for a compromise solution between the complexity and the measure of approximation.

- An other parameter is the maximum number of function evaluations in the optimization function “fminsearch”. By begin is advised to choose 20-50 steps with follow-up increasing when the model structure is satisfactory.

- For model verification is necessary to move the model output to the working point.

    - Experimental identification procedure

The procedure of experimental identification of a system by searching the best model can be described by a flow-process diagram as it is shown in Fig. 3.10.  Result from diagnostic evaluation and verification can optimize the model structure and the design of identification measure in feedback.

Completion

Model Verification

Plant Experiment

Determine Model Structure

Data Processing-Parameter Estimation

Design of Identification Measure

Diagnostic Evaluation

A priory

knowledge

Alternative

data

Feedback Model

Optimization

Fig. 3.10 Experimental identification procedure

  - Movement to the working point

By the model verification of congruence the outputs between the model yM(t) and planty(t)are checked. Therefore it is suitable to move the linear model output ∇yM(t) to the working pointy0,u0.The parameter estimation in the fig.3.8 has inputs  ∇u(t)=u(t)−u0  and ∇y(t)=y(t)−y0. The output from the parameter estimation  (identification) as a model is the transfer function F(s). The linear (LTI) model input is ∇uM(t)=∇u(t)=u(t)−u0 and

a)

b)

Obr.3.9 Variants of movement to the working point

d)

c)

Σ

Σ

the output is ∇yM(t)(see Fig. 3.9a). From the transfer function F(s) the gain of the model KMcan be evaluated. There are some variants how the movement can be done to the working point.

  - According the definition of the transfer function

  - When we will carry out the moving in the working point, the input to the LTI model must be ∇uM(t)=∇u(t)=u(t)−u0  and the output will be ∇yM(t). In working point the output from model will be yM(t)=∇yM(t)+y0 as shown in the Fig. 3.9b.

  - An other possibility how to carry out the moving to the working point is shown in Fig. 3.9c. By this structure the addition of y0 to output will be replaced by the term uKOR=y0KM on input. The LTI model input can be calculated as

uM(t)=u(t)−u0+uKOR=u(t)−u0+y0KM=∇uM(t)+y0KM

  - In the Fig. 3.9.d  is shown the variant, which has the input to the LTI model  u(t) . It means that the influence of u0 must be replaced on LTI output as the term  yKOR=−u0.KM . The model output can be calculated as

yM(t)=∇yM(t)+y0−yKOR=∇yM(t)+y0−u0.KM

## Model Reduction

By the experimental identification the order of the mathematical model and the structure of the transfer function (order of the polynomial in numerator and denominator) must be estimated.

Basic steps and the procedure will be shown by the following example. Here's shown how to reduce the model "by hand" without the use of MATLAB software resources.

Example 3.8-1

Let us consider a SISO system, which dynamic is approximated by the transfer function:  F(s)=1s+0,01s+0,2s+5.

Reducing the model order can be performing by residualisation or truncation.

a) Residualisation

The transfer function can be written in the form

1s+0,01s+0,2s+5=0,1s+0,01⋅2s+0,2⋅5s+5

You can see that the factor  5s+5 has the mode (root) that represents a mode, which in comparison with the remaining modes is very fast, so that its dynamic can be neglected.

We therefore consider only steady state for which the factor is equal to 5s+5≈1. Reduced transfer function (model) is equal to   FR(s)=0,1s+0,01⋅2s+0,2.

b) Truncation

Consider we write F(s) in terms of partial-fraction expansion:

1s+0,01s+0,2s+5=1,0547s+0,01−1,0547s+0,2+0,0418s+5

You can seen that the gain of the third member in comparison with other is small, so that we can consider: 0,0418s+5≈0. Reduced transfer function is than equal

FR(s)=1,0547s+0,01−1,0547s+0,2=0,200394s+0,01s+0,2.

End of example 3.8-1

## Standardization of input and output signals

By analyzing and simulation of the model properties of real systems it is usually necessary to carry out a standardization of input and output signals. The aim of standardization is that the inputs/outputs were approximately equal, and lay in the interval (-1, +1). Standard output is equal to

y∗(t)=S0⋅y(t)→Y∗(s)=S0⋅Y(s),

where   S0  … is a diagonal matrix of conversion factors (coefficients) of output signals .

In a similar way for standardized output can be written

u∗(t)=Sι⋅u(t)→U∗(s)=Si⋅U(s),

where Si…. is a diagonal matrix of conversion factors (coefficients) of input signals.

Since the Laplace transform of output Y(s) can be calculated from the transfer function and L transform from input, the standardized output is equal to

Y(s)=F(s)U(s)→Y∗(s)=S0F(s)U(s).

In a similar way the standardized input can be calculated and than the normalized output is equal to

U(s)=Si−1
              ⋅U∗(s)→Y∗(s)=S0F(s)Si−1
              ⋅U∗(s)=F∗(s)⋅U∗(s)

Normalized transfer matrix (transfer function) matrix is calculated from the equation

F*(s)=S0F(s)Si−1
            .

- Tools for parameter estimation

  - System Identification Toolbox “Ident”

  - MATLAB Program for parameter estimation

### Flow-process diagram

Data Enter (File name, measured data,

data variable, working point)

Start

Evaluation of the criterion

’critT’

Creating   u, y, tG and

increments uG, yG

Plot Graph

Displaying the transfer

function structure

Creating of the vector x and a LTI object.

Settings OPTIONS value.

Invoke „fminsearch (‘crit‘, x, OPTIONS)

OUTPUTS

End

### Program and function in MATLAB

%A_idT

%F=K/(Ts+1)^n, n=3,  measure with sample time=0,1sec

close all

clear all

global tG uG yG N A

load ('SU02')               %Data Enter,Datei name, measured data,

%data variable

u=W(:,1);y=W(:,2);          %Creating of the vector u, y

tG=0:0.1:(length(u)-1)*0.1; %Creating of the time vector tG

figure (1)

plot(tG,y,tG,u)             %Plot graph - measured data

title ('Graph of mesurenment')

%working point

y0=2.5;u0=2;                %working point

yG=y-y0;                    %increment yG

uG=u-u0;                    %increment uG

figure (2)

plot(tG,yG,tG,uG)           %Plot graph - increments yG, uG

title ('Graph of increments yG,uG')

pause

%Displaying the transfer function structure

disp('Approximation by the transfer funcion')

disp('          x(1)                K ')

disp('F(s)=------------------ = -----------')

disp('      (x(2).s + 1)^3      (Ts + 1)^3')

%Creating of the vector x and a LTI object

N=2                 %N=n-1

T=0.5               %start at the Time constant=0,5

K=1                 %start at the gain=1

A1=[T 1];           %polynomial A1=(Ts+1)

A=A1;

for i=1:N;          %evaluation of (Ts+1)^3

A=conv(A,A1);

end

s=tf(K,A)           %LTI object s

roots(A)

x=[K T]             %start point x0:x(1)=K…gain, x(2)=T… time constant

critT(x)            %evaluation of the function to be minimzed

disp('running...')

%setting the values of the field in the "options" structure

OPTIONS=optimset('MaxIter',50,'TolX',1e-1);

%start of the finding a minimum by the "fminsearch"

x=fminsearch('critT',x,OPTIONS);

%outputs

disp('Optimalized vector x')

x

disp('Function/criterion value J')

critT(x)

roots(A)

tf(x(1),A)

sys=tf(x(1),A);

[yi,ti]=lsim(sys,uG,tG);

figure(3)           %Plot graph outpu of the LTI model and measured increments

plot(tG,yG,tG,uG,ti,yi)

The criterion as an M-file “criT”

%criT(x)

function f=critT(x)     %the function to be minimzed

global tG yG uG N A

A1=[x(2) 1];

A=A1;

for i=1:N;              %evaluation of the polynomial [x(2)+1]^3=(Ts+1)^3

A=conv(A,A1);

end

sys=tf(x(1),A);         %creating of a LTI object

[yi,ti]=lsim(sys,uG,tG);%evaluation of yi=yM(i)

%plot(ti,yi,ti,yG)

%pause (0.1)

f=sum((yG-yi).*(yG-yi));%function evaluation

Summary

In the chapters1- 4 we have introduced the concepts of identification of a dynamic system based on measured input/output. The typical identification process consists of stages where you iteratively select a model structure, compute the best model in the structure, and evaluate this model’s properties. This cycle can be itemized as follows:

- Design an experiment and collect input/output data from the process to be identified.

- Examine the data. Polish the data by removing trends and outliers

- Select and define a model structure (a set of candidate system descriptions)

- Compute the best model in the model structure according to the input/output data and given criterion for goodness of fit

- Examine the model properties of the obtained model

- If the model is good enough, then stop; otherwise go back to step 3 to try another model structure or go back to 1 to find better input/output data for identification.

- Introduction to using MATLAB

Subject specifications consider that utilizations of MATLAB software tools are involved in practical lessons.  As you know MATLAB is a technical computing language for high-performance mathematical computation and visualization. MATLAB integrates numerical analysis, matrix computation, signal processing, and graphics in easy to use environment.

The MATLAB R2006a involves products of the MATLAB Product Family. The structure can be depicted as shown in the Fig.5-1. You see the MATLAB consists of MATLAB Extension, MATLAB Toolboxes, SIMULINK, SIMULINK Extension and SIMULINK Blocksets.

- MATLAB Extension are optional tools that supports the implementation of systems developed in MATLAB.

- MATLAB Toolboxes are libraries of MATLAB functions and tools.

- SIMULINK is a system for nonlinear simulation that combines a block diagram interface and simulation with numeric, graphics, and language functionality of MATLAB.

- SIMULINK Extensions are optional tools that support the implementation of systems developed in SIMULINK.

- SIMULINK Blocksets are collections of SIMULINK Blocks designed for use in specific  application areas.

The software product is very good described in user’s manuals [1,2,3,4,5] and offers help functions by implementation of this software.

In spite of that it is not easy for students to use it because of the software is very large.

Students must apply this software for solving theoretical and practical tasks. To help them to find quickly orientation in MATLAB software we prepared in chapter 5 an introduction to MATLAB. Of course in this chapter we explain only the most important functions that are necessary to understand explained identification program and carry out necessary changes.

We hope that shall help students to be able as soon as possible to solve their tasks.

SIMULINK

MATLAB

MATLAB Extension

MATLAB Compiler

MATLAB C Math Library

MATLAB Toolboxes

Control Systems

Financial

Fuzzy Logic

Frequency Domain System

Financial

Model Predictive Control

System Identification

Model Predictive Control

-Analysis and Synthesis

Neural Network

Optimisation

Robust Control

Signal Processing

Spline

Statistic

Symbolic Mat

SIMULINK Extensions

Real Time Workshop

SIMULINK Block sets

Fig.5-1. The MATLAB Product Family

Command

Windows

WORKSPACE

Variables

Files

Objects

  - Desktop Tools and Development Environment

- Starting MATLAB

On Windows platforms, start MATLAB by double-clicking the MATLAB shortcut icon on your Windows desktop.

You can customize MATLAB startup. For example, you can change the directory in which MATLAB starts or automatically execute MATLAB statements in a script file named startup.m. For More Information    See Starting MATLAB in the Desktop Tools and Development Environment documentation.

- Quitting MATLAB

To terminate your MATLAB session, select File -> Exit MATLAB in the desktop, or type quit in the Command Window. You can run a script file named finish.m each time MATLAB quits that, for example, executes functions to save the workspace, or displays a quit confirmation dialog box. For More Information    See Quitting MATLAB in the Desktop Tools and Development Environment documentation.

- MATLAB Desktop

When you start MATLAB, the MATLAB desktop appears (see Fig.5.1-1), containing tools (graphical user interfaces) for managing files, variables, and applications associated with MATLAB. The illustration in Fig. 5.1-1 shows the default desktop. The MATLAB desktop consists of Command Window, Current Directory and Command History window.

You can customize the arrangement of tools and documents to suit your needs. For more information about the desktop tools, see Desktop Tools and Development Environment.

Fig. 5.1-1 MATLAB desktop

  - Workspace, Editor, Search path and File Operations

The workspace is the set of variables maintained in memory during a MATLAB session. Use the Workspace browser or equivalent functions to view the workspace.

### Workspace operations

This function displays Workspace browser, the   graphical user interface that allows you to view and manage the contents of the  workspace.

workspace

who, whos

- This functions list all variables in the currently active workspace.

clear all

3)			      This function removes all variables from the workspace.

clear name

4)			      This function removes just the M-file or MEX-file function or

    - variable name from the workspace.

clear global name

5)				  This function removes the global variable name.

### Editor and Command Window operation

clc

1)			      This function clears the screen.

edit

2)			      This function opens a new editor window.

This function opens the M-file fun in the default editor.

edit fun.m

edit fun1 fun2 …    …

- This function opens fun1.m fun2.m, and so on, in the

default editor.

You can use an alternative to the “edit” function, select New or Open from the File menu in the MATLAB desktop or any desktop tool.

### Search part and File operation

search

- This function displays the current MATLAB search path.

cd

- This function changes working directory.

This function lists the directory.

dir

exist name

- This function checks existence of variable, function,

or directory.

what

- This function lists the M, MAT, MEX, MDL, and P-files and the class directories that reside in the current working directory.

which fun

- This function displays the full pathname for the argument fun.

type ('filename')

type filename

- This function displays the contents of the specified file in the MATLAB Command Window.

### Select function size, length, roots, conv, disp, pause

d=size (X)

- This function returns the sizes of each dimension of array X in

a vector d with ndims(X) elements. If X is a scalar, which

MATLAB.

[m n]=size (X)

- This function returns the size of matrix X in separate variables m

and n.

This function returns the size of the longest dimension of X. If X is

n = length(X)

a vector, this is the same as its length.

This function returns a column vector whose elements are the

r = roots(c)

roots of the polynomial c.

w = conv(u,v)

- This function convolves vectors u and v. Algebraically, convolu

tion is the same operation as multiplying the polynomials whose

coefficients are the elements of u and v.

disp (X)

6)                                  This function displays an array, without printing the array name.

If X  contains a text string, the string is displayed.

pause

- This function by itself, causes M-files to stop and wait for you to

any key before continuing.

8)                                  This function pauses execution for n seconds before continuing,

pause(n)

where n can be any nonnegative real number. The resolution of

the clock is platform specific. A fractional pause of 0.01 seconds

should be supported on most platforms.

  - LTI models

    - Creating of Some Linear Models

1) Creation of transfer function models

Specify transfer function by given coefficient of nominator or denominator, or converts state-space or zero-pole-gain

Function tf

models  to the transfer function form.

Syntax function

sys = tf(num, den)

tfsys = tf(sys)

Creates a continuous-time transfer function with numerator(s)

and denominator(s)

Converts an arbitrary SS or ZPK LTI model sys to transfer

function form

where

sys 			is a TF object storing the transfer function data,

num, den		are the real- or complex-valued row vectors of numerator and

denominator coefficients ordered in descending powers of.

These two vectors need not have equal length and the transfer

function need not be proper. For example, s1=tf([1],[1 0] speci

fies the integrator 1/s.

2) Creation of zeros-pole-gain model

Function zpk

Specify zero-pole-gain model or convert LTI model to zero-pole.

Syntax function

sys = zpk(z,p,k)

zsys = zpk(sys)

creates a continuous-time zero-pole-gain model with ze-

ros z, poles p, and gain(s) k.

converts an arbitrary LTI model sys to zero-pole-gain

form

where

sys			is a ZPK object storing the model data

z and p 			are cell arrays of vectors with as many rows as outputs and as

many columns as inputs, and k is a matrix with as many rows

as outputs and as many columns as inputs.

In the SISO case, z and p are the vectors of real- or complex-valued zeros and poles, and k is the real- or complex-valued scalar gain.

sys=k(s−z(1))(s−z(2))...(s−z(m))(s−p(1))(s−p(2))...(s−p(n))

Set z or p to [] for systems without zeros or poles. These two vectors need not have equal length and the model need not be proper (that is, have an excess of poles).

3) Creation of State-Space Models

Function ss

specify state-space models or convert LTI model to state

space

Syntax function

sys = ss(A,B,C,D)

sys_ss = ss(sys)

creates the continuous-time state-space model,

converts an arbitrary TF or ZPK model sys to state space.

In the SISO case, for a model with Nx states x, one output yand one input u , the continuous state space model can be described by equations

x=Ax+Buy=Cx+Du

where

A	is an [Nx,Nx] real- or complex-valued matrix

B	is an [Nx,1] real - or complex-valued vector

C	is an [1,Nx] real - or complex-valued matrix

D	is an [1,1] real- or complex-valued scalar. If you can simply set d to zero.

sys	is an SS model that stores the model data

Example  5.3-1

Example 5.1-3

Example  5.1-2

Example  5. 3- 4

Example 5. 3-5

    - Data Extraction

Function tfdata

1) 				  Access transfer function data

Syntax function

[num,den] = tfdata (sys)

[num,den] = tfdata (sys,‘v‘)

returns the numerator(s) and denominator(s) of the transfer function as a LTI array of , SS or ZPK models,

For SISO transfer function, the syntax forces tfdata to

return the numerator and denominator directly as row

vectors rather than as cell arrays.

Example  5. 3-7

Example 5. 3- 6

Function zpkdata

2) 	                                                  Access zero-pole-gain data

Syntax function

[z,p,k] = zpkdata (sys)

[z,p,k] = zpkdata (sys,‘v‘)

The syntax returns zeros z, poles p, and gain(s) k of the zero- pole-gain model sys.

For SISO transfer function, the syntax forces zpkdata to

return the zeros and poles directly as column vectors

rather than as cell arrays (see example below).

Example  5. 3- 8

Example  5. 3- 9

Function ssdata

3) 	            sss                                      Access state-space data

Syntax function

[a,b,c,d] = ssdata (sys)

The syntax extracts the matrix (or multidimensional array) data  from the state-space model (LTI array) sy

Example  5. 3-10

Example  5. 3- 11

  - Time Domain Analysis

Function step

1) 				 		 Step response of LTI systems

Syntax function

This step function calculates the unit step response of a

step (sys)

step (sys,t)

step (sys1,sys2,…,sysN)

[y,t,x] = step (sys)

linear system.

When invoked with no output arguments, this function

plots the step response on the screen.

This function sets the simulation horizon explicitly. You

can specify either a final time t = Tfinal or a vector of

evenly spaced time samples of the form t = 0:dt:Tfinal

This function plots the step responses of several LTI mod-

els.

This syntax returns the output step response y, the time

vector t used for simulation, and the state trajectories x

(for state-space models only).

Example  5.4- 1

Function impulse

2) 				 		 Impulse response of LTI systems

Syntax function

This impulse function calculates the unit step response

impulse (sys)

impulse (sys,t)

impulse (sys1,sys2,…,sysN)

[y,t,x] = impulse (sys)

of a linear system.  When invoked with no output arguments, this function plots the impulse responses on the screen.

This function sets the simulation horizon explicitly. You

can specify either a final time t = Tfinal or a vector of

evenly spaced time samples of the form t = 0:dt:Tfinal

This function plots the impulse  responses of several LTI

models.

This syntax returns the output impulse response y, the time vector t used for simulation, and the state trajectories x (for state-space models only).

Example  5. 4- 2

Function lsim

3) 				 	 Simulate LTI model responses to arbitrary inputs

Syntax function

This function simulates the (time) response of continuous or discrete linear systems to inputs u and time history t. The vector t specifies the time samples for the simulation and consists of regularly spaced time samples: t = 0:dt:Tfinal.

lsim (sys,u,t)

lsim (sys,u,t,x0)

[y,t] = lsim (sys,u,t)

[y,t,x] = lsim (sys,u,t)

When invoked without left-hand arguments, lsim plots the response on the screen.

This function further specifies an initial condition x0 for the system states.

This syntax returns the output response y, the time vector t used for simulation. No plot is drawn on the screen.

This syntax returns the output response y, the time vec-

tor t used for simulation, and the state trajectories x for

state-space models only.

lsim(sys) opens the Linear Simulation Tool GUI. For more information about working with this GUI, see Working with the Linear Simulation Tool in

Function gensig

4) 				 	 Generate test input signals for lsim

Syntax function

[u,t]=gensig (type,tau)

[u,t]=gensig (type,tau,Tf,Ts)

This function generates a scalar signal u of class type

with period tau (in seconds) and returns a vector t of

time samples. The vector u of signal is defined values

at these samples. All generated signals have unit ampli

tude.The following types of signals are available:

'sin'     	Sine wave

'square'	Square wave

'pulse'		Periodic pulse.

This function also specifies the time duration Tf of the

signal and the spacing Ts between the time samples t.

You can feed the outputs u and t directly to lsim and simulate the response of a single-input linear system to the specified signal.

Example  5. 4- 3

  - Some basic plotting function

MATLAB provides a variety of functions for displaying vector data as line plots, as well as functions for annotating and printing these graphs. We will explain the function “plot” that produces 2-D line plots with linear scales for both axes

Function plot

1)			 		 2-D line plot

Syntax function

This function  plots the columns of Y versus their index if Y  is a real number.

plot (Y)

plot (X1,Y1,…)

This function plots all lines defined by Xn versus Yn pairs

Example  5.5-1

Function title

2)			 		 adds title at the top of the Figure

Syntax function

title (‘string‘)

This function adds the string at the top of the figure

Function xlabel, ylabel, zlabel

3)

Syntax function

xlabel (‘string‘)

ylabel (‘string‘)

zlabel (‘string‘)

This function labels the x-axis of the current axis

This function labels the y-axis of the current axis

This function labels the z-axis of the current axis

Example  5. 5-2

Function LineSpec

- determines line type, marker, symbol and color

of the plotted lines.

MATLAB enables to define Line style, Marker type, Color, Line width, Marker size, Marker face and edge coloring. MATLAB uses string specifiers for Line style, Marker type and Color that are listed in Tab.1,2 and 3:

Tab.1 Line Style specifiers 			Tab.2   Marker Specifier

| Line style | Specifier |
| --- | --- |
| solid line | - |
| dashed line | -- |
| dotted line | : |
| Dashed-dot line | -. |

| Marker Type | Specifier |
| --- | --- |
| Plus sign | + |
| Circle | o |
| Asterisk | * |
| Point | . |
| Cross | X |
| Square | s |
| Diamond | d |
| Upward-pointing triangle | ^ |
| Downward-pointing triangle | V |
| Right-pointing triangle | > |
| Left-pointing triangle | < |
| Five-pointed star | p |
| Six-pointed star | h |

Tab.3 Color Specifiers

| Color | Specifier |
| --- | --- |
| Red | r |
| Green | g |
| Blue | B |
| Cyan | c |
| Magenta | m |
| Yellow | y |
| Black | k |
| White | w |

Function figure

4)			 		 Create figure graphic object

Syntax function

figure

figure (h)

This function creates a new figure objects - the individual windows on the screen

Suppose h is an integer and is not a number of existing figure, then this

function  creates a figure and assigns it the number h.

Function hold

5)			 	Retain current graph in figure

Syntax function

hold

hold on

hold off

hold all

This function determines whether new graphics object are added to the graph or replace objects in the graph.

This function retains the current plot and certain axes properties so    that subsequent graphing commands add to the existing graph.

This function resets axes properties to their defaults before drawing

new plots

This function holds the plot and the current line color and line style so

that subsequent plotting commands do not reset the ColorOrder and

LineStyleOrder property.

Applying of the function “figure” and “string specifiers” for line style and color is presented in the example 5.3-3. In the Figure 1 only the color is specified. The first string defines a red color   ‘r’, the second string the blue color ‘b’.

In the Figure 2 is added the specification of the Line style. The first string defines a dashed line of red color   ‘--r’, the second string the dashed-dot line of blue color. The first string defines a dashed line of red color   ‘--r’, the second string the dashed-dot line of blue color ‘-.b’.

Example  5. 5-3

Use of the string specifiers for marker, line and color is shown in the example 5.3-4. In the figure 3 the first string defines a dashed line of red color with marker type of square ‘--rs’, the second string the dashed-dot line of blue color with marker type of circle ‘-.bo’.

Example  5. 5-4

The other line characteristic can be specified as Related Properties by follow strings:

‘LineWidth’		… the width in points of the line,

‘MarkerEdgeColor’	… the color of the marker or the edge color for filled marker,

‘MarkerFaceColor’	… the color of the face of filled marker

‘MarkerSize’		… the size of the marker,

Use of the function “hold” and strings ‘LineWidth’, ‘MarkerEdgeColor’, ‘MarkerFaceColor’,‘MarkerSize’ is demonstrated in the example 5.3-5.

In the figure 1 is first prepared for plot the line 1. We added to the line specification the specification of the line width 'LineWidth',2,, color of the marker 'MarkerEdgeColor','k',color of the face of the of filled marker 'MarkerFaceColor','g', and the size of the marker 'MarkerSize',10. The marker size of the square is 10 points.

After setting the command “hold on”  is added the line 2. The line width is now 4 points, the marker size of the circle is 20 points

Example  5.5-5

  - Opening, loading, saving files

Function save

1)			 	Save workspace variable to disk

Syntax function

This function stores all workspace variable in a binary

save

save filename

save filename var1 var2…

save (filename)

save (’stringfilename’)

save (‘filename’,’var1’,…)

format in the current directory in a file named

“matlab.mat”. Retrieve the data with   “load”.

This function stores all workspace variables in the current directory in the “filename.mat”.

This function stores only specified workspace variables var1,var2,… in the given filename.mat”.

This functions store all workspace variables in the

current directory in the “filename” that is a string.

You must specified: filename=’stringfilename’

You can use direct the string ’stringfilename’

This function stores only specified workspace variables described as string ‘var1’,… in the given ‘filename’.

Save all variables from workspace in binary MAT-file

Example  5.6-1

named matlab.mat. List variables in workspace applying function whos. Check files in current directory using functions what.

Save all variables from the workspace in binary MAT-file

Example  5. 6-2

named 14july.mat. List files in current directory using functions

what.

Example  5. 6-3

a)  Save the y, t variables from the workspace in binary MAT-file

named 14july1.mat.

- Assign the string ‘test14_july2’to the variable – filename savefile. Save all variable in file specified by string ‘test14_july2’

- Save the variable A, B specified by strings ‘A’, ‘B’ in the file specified by the string ‘test14_july1’.

- Save all variables from the workspace to the binary MAT-file

named by the string ‘test14_ july3’.

- Save the variables specified by strings ‘y’, ‘t’ from the workspace in the MAT file named by the string ‘test14_july4’. List files in current directory the using function what.

a)

b)

c)

Example  5.4-1

d)

e)

d)

e)

Function load

2)			 	Load to workspace variable from disk

Syntax function

This function loads all the variables from the MAT-file mat in  “matlab.mat” to the workspace.

load

load filename

load filename X Y Z …

load filename –regexp expr1 expr2 …

load –asci filename

load –mat filename

This function loads all variables from the file specified by  the “filename.mat” to the workspace. The filename is a string specifying by a file name.

This function loads just the specified variables X, Y, Z, etc. from the MAT-file.

This function loads just those variables that match any of the regular expressions given by expr1, expr2, etc.

This function forces load to treat the file as an ASCI- file, regadless of file extension. If the file is not numeric text, load returns an error.

This function forces load to treat the file as an MAT- file, regadless of file extension. If the file is not numeric text, load returns an error.

Example  5. 6-4

Loading from a binary MAT-file.

      - Use function who to see which variables are in the work space.

      - Clear workspace and load a binary MAT-file specified by its string. Check the variables in the Workspace.

Example  5.4-4

Clear the workspace and load the variables from the MAT-file 14july. Check the variables in the Workspace.

Example  5. 6-5

- Clear the workspace and load the variables from the MAT-file 14july1. Use the function who to see which variables are in the work space.

Example  5. 6-6

Loading a list of variables.

- Clear the workspace and load the variables y t from the MAT-file 14july. Use the function who to see which variables are in the work space.

- Clear the workspace and load the variable y from the MAT-file 14july. The file and variable y are specified as a string. Use the function who to see which variables are in the work space.

  - Programming-flow control

Function If, else, and elseif

1.

The if statement evaluates a logical expression and executes a group of statements when the expression is true. The optional elseif and else keywords provide for the execution of alternate groups of statements. An end keyword, which matches the if, terminates the last group of statements. The groups of statements are delineated by the four keywords -- no braces or brackets are involved.

Example  5.7-1

Syntax function

if expression1

statements1

else if expression2

statements2

else

statements3

end

Expression is a MATLAB expression, usually consisting of variables or smaller expressions joined by relational operators (e.g., count < limit), or logical functions (e.g., isreal(A)).

Simple expressions can be combined by logical operators (&&, ||, ~) into compound expressions such as the following. MATLAB evaluates compound expressions from left to right, adhering to operator precedence rules.

(count < limit) && ((height - offset) >= 0)

Statements is one or more MATLAB statements to be executed only if the expression is true or nonzero.

Function switch and case

2.

The switch statement executes groups of statements based on the value of a variable or expression. The keywords case and otherwise delineate the groups. Only the first matching case is executed. There must always be an end to match the switch

Switch statements syntax

switch switch_expr

case case_expr

statement, ..., statement

case {case_expr1, case_expr2, case_expr3, ...}

statement, ..., statement

otherwise

statement, ..., statement

end

The switch statement syntax is a means of conditionally executing code. In particular, switch executes one set of statements selected from an arbitrary number of alternatives. Each alternative is called a case, and consists of The case statement One or more case expressions One or more statements

In its basic syntax, switch executes the statements associated with the first case where switch_expr == case_expr. When the case expression is a cell array (as in the second case above), the case_expr matches if any of the elements of the cell array matches the switch expression. If no case expression matches the switch expression, then control passes to the otherwise case (if it exists). After the case is executed, program execution resumes with the statement after the end.

Example  5.7-2

Function for

3.

The for loop repeats a group of statements a fixed, predetermined number of times. A matching end delineates the statements.

The columns of the expression are stored one at a time in the variable while the following statements, up to the end, are executed. In practice, the expression is almost always of the form scalar : scalar, in which case its columns are simply scalars. The scope of the for statement is always terminated with a matching end:

Example  5.7-3

Syntax

for variable1 = expression1

statements

...

for variable2 = expression2

...

statements

...

end2

...

statements

...

end1

4.

Function while

The while loop repeats a group of statements an indefinite number of times under control of a logical condition. A matching end delineates the statements.

while expression

statements

end

while repeats statements an indefinite number of times. The statements are executed while the real part of expression has all nonzero elements. expression is usually of the form

expression rel_op expression

where rel_op is ==, <, >, <=, >=, or ~=.

The scope of a while statement is always terminated with a matching end.

Function continue

5.

The continue statement passes control to the next iteration of the for loop or while loop in which it appears, skipping any remaining statements in the body of the loop. In nested loops, continue passes control to the next iteration of the for loop or while loop enclosing it.

Function break

6.

The break statement lets you exit early from a for loop or while loop. In nested loops, break exits from the innermost loop only

Function return

7.

The return terminates the current sequence of commands and returns control to the invoking function or to the keyboard. return is also used to terminate keyboard mode. A called function normally transfers control to the function that invoked it when it reaches the end of the function. You can insert a return statement within the called function to force an early termination and to transfer control to the invoking function.

# Literatura

[ 1] NOSKIEVIČ,P.: Modelování a identifikace systémů. MONTANEX a.s.,

1999, Ostrava.

[ 2] GRACE, A.-LAUB, J.A-LITTLE, J.N.-THOMPSON, C.M.: Control System Tool-

box. For Use with MATLAB. User's Guide. The Math Works, Inc.1995.

[ 3]  HANUŠ, B.- ALEXÍK, M.- BÍLA, J.- KROC, L.- KRUPKA, Z.: Teorie automatického

řízení II. Skripta Liberec,1985.

[ 4] NELDER, J.A.-MEAD, R.: A Simplex Method for Function Minimization. Cmputer

Journal, Vol.7, p.308-313.